# Texas 2D BLE Analysis QA/QC Checklist

# Project Information

| HUC-8 Watershed Name & ID: | Toyah: 13070003 and Barilla Draw: 13070005 |
| --- | --- |
| HUC-10 Watershed Name & ID: |  |
| Modeler: | Watearth/AtkinsRealis |
| Reviewer: | AECOM |

## M1 Review: Terrain, Modeling and Preliminary Tie-In Review (QC Items 1-6)

| Modeler Initials: | SGK/CQ | Reviewer Initials: | JW |
| --- | --- | --- | --- |
| Submittal Date: | May 1, 2024 | Review Submitted Date: | 5/20/2024 |
| Review Response Date: | 6/25/2024 | Backcheck Complete Date: | 6/28/2024 |
| Review Response Date2: | 7/19/2024 | Backcheck2 Complete Date: | 7/26/2024 |
| Review Response Date 3: | 8/22/2024 | Backcheck3 Complete Date: | 8/26/2024;10/18/2024 |

## M2 Review: Mapping and Completeness Review (QC Item 7)	submittal

| Modeler Initials: | SGK/CQ | Reviewer Initials: | ES |
| --- | --- | --- | --- |
| Submittal Date: | 10/08/2024 | Review Submitted Date: | 10/18/2024 (sent 12/9/24 - SS) |
| Review Response Date: | 4/23/25 | Backcheck Complete Date: | 5/7/2025 |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X | 2D BLE |  | X | 10% ACE (10-year) | X | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | X | 4% ACE (25-year) | X | 1% plus |
|  |  |  | X | 2% ACE (50-year) | X | 1% minus |
|  |  |  | X | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

Reviewer to indicate if Technical Review items generally comply using a “Pass”. If the technical review observes an issue of noncompliance or an item that requires additional attention, an “Fail” should be utilized.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| QA/QC Comments Addressed |  |
| --- | --- |
|  | M1 Comments have been addressed, we understand the comments are an example and not comprehensive of all the errors in the watershed. We have reviewed the entire HUC based on the QA/QC and revised our model accordingly. |
|  | M2 Comments have been addressed, we understand the comments are an example and not comprehensive of all the errors in the watershed. We have reviewed the entire HUC based on the QA/QC and revised our mapping accordingly. |
| M1: Base Map Data (QC items 1-6) |  |
| x | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference) |
| X (RAS Mapper) | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| M1: Technical Report / Documentation (QC items 1-6) |  |
| x | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
| x | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| x | Internal QA/QC Documentation: comments and responses |
| x | Flow comparison tables within the technical report |
| M1: Hydrology (QC items 1-6) |  |
| x | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
| x | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
| x | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| M1: Hydraulics (QC items 1-6) |  |
| x | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events run and results available in RASmapper (remove non-final files) Exported Raster Grids for 10, 100-, 500 yr events (using sloping render mode, unless previously coordinated with TWDB) Difference Raster for tie-ins? (for 10-, 100, 500yr events) (add some descriptions) |
| x | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
| x | Manning’s n-value table |
| x | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
| x | Hydraulic modeling notebook including assumptions, notes, and special conditions (Draft BLE Report with sections related to terrain, modeling and validation) |
| M2: Mapping (QC item 7) |  |
| x | Flood risk database |
| x | Modeler to complete topology validation as outlines in the guidelines |
| x | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| M2: Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
| X | Please submit items using the eBFE USGS format and MIP Terrain Folder (see example folder in BLE Resources) |
| X | Final Technical BLE Report |
| X | Final cleaned hydrologic model |
| X | Final cleaned hydraulic model |
| X | Flood risk database |
| X | CNMS updates |
| X | HAZUS results |
| x | Additional supplemental data: 1) ArcHydro outputs (if applicable), 2) Model domain shapefile, 3) LiDAR Extents Shp file, |

Milestone 2 note to production team:  Please complete the info on Page 1 and the Submittal Checklist above. Looks like this has not been updated for M2 Reviews.

Response 20250423: This has been updated

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): | Please see draft BLE report for more detail |
| --- | --- |
| Data Gaps: | None |
| Source Resolution: | Cell size of 1 x 1 meter |
| Model Terrain Resolution: | Cell size of 10 x 10 feet |
| Terrain Horizontal Projection: | North American Datum (NAD) 1983 Texas State Plane Central FIPS 4203 |
| Terrain Vertical Projection: | NAVD88 |
| Z Values: | Units of feet |
| Terrain Buffer: | 2,500 feet |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| General Review |  |
| --- | --- |
| F P | LiDAR source matches the latest available data at time of project initiation |
| P | Large reservoir or lake extents included in model domain |
| P | Check visible seams, gaps or holes of LiDAR sources (if more than one source was used) |

## Review Comments

| Modeler to Check Box of Acknowledgement during Response to Comments: | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

- There are 5 terrain modifications used (TerrainMod_1, TerrainMod_2, etc.) that seem to be trying to correct unusual high ground strips in the terrain. The terrain mods don’t have elevations set and don’t seem to actually be correcting anything. Please provide more documentation on the addition of these terrain mods and how/why they’re being applied.

Watearth Response (5/24/2024):  There were mining sites/borrow pits present in several of the model domains. The sites included large depressions that were typically 30 to 40 feet deep and up to one mile wide. In some instances, the sites were located near streams and had elevations lower than the stream channel’s thalweg. The terrain modification polygons are used to adjust these elevations, and they are removing the mines and borrows from the actual terrain. Below the actual terrain and the terrain with modifications are shown for the same area:

With terrain modification:

Without terrain modification:

- Backcheck response (7/09/2024): Comment has passed.

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.4.1 |
| --- | --- |
| Rainfall Source: | NOAA Atlas 14 |
| Rainfall Duration: | 24 hours |
| Depth Selection: | Domain Average |
| Rainfall Temporal Distribution: | NRCS |
| Areal Reduction: | None |
| Loss Methodology: | Spatially varied CN |
| Unit Hydrograph Transform Method: | N/A |
| Inflow Hydrograph Source: |  |
| Any Deviations from TWDB Guidance? If yes, please describe |  |
| Other: |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: | No specific gage in this model domain. Multiple |
| --- | --- |
| Stream Gage Name & ID: |  |
| Stream Gage Name & ID: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Excess Rainfall |  |
| --- | --- |
| P | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event or other appropriate rainfall application |
| P | CN values appropriately reflect land use and soils for entire 2D area boundary |
| P | Losses are correctly calculated to establish the excess rainfall hyetographs by checking land use, soils and infiltration layers or other relative information |
| P | Total precipitation depths for 1%+ and 1%- are calculated correctly |
| P | Excess rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) |
| P | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |
| P | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |
| Inflow Hydrographs |  |
| N/A | Shape and time-to-peak of any inflow hydrographs resemble past documented large storm events |
| N/A | Hydrographs reasonable when compared to the other return periods. The shapes should be similar, and hydrographs from different events should not intersect unless there is an engineering reason. Peak flows from a lower return period event should not be higher than a higher return period event |
| N/A | Contributing hydrographs from another RAS model applied at coincident locations (Provide locations in base map data) |
| General Hydrology |  |
| No | Any flow regulating dams in the model area? (Yes/No) |
| No | Dam modeling methodologies applied and explained? (Yes/No). If yes, provide discharge rating curve calculations and gage analysis documenting regulated versus unregulated records, if applicable. |
| N/A | Model aligns with documentation of dam modeling approaches. |
| Other Notes: NA |  |

## Review Comments

Modeler to Check Box of Acknowledgement during Response to Comments:

| X | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.4.1 |
| --- | --- |
| Loss Methodology: | Spatially varied CN |
| Soils Source: | SSURGO |
| Land Cover Source: | NLCD 2021 |
| Manning's Roughness Source: | Assigned based on land use. |
| Nominal Mesh Cell Sizes: | 300 x300 except for channel refinement regions, breaklines and urban regions, where 150 ft x150 ft was considered. |
| Any deviations from TWDB Guidance? If yes, please describe. |  |
| Other: 0.3 sq. mi stream break lines used for building mesh geometry |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| 2D Mesh |  |
| --- | --- |
| P | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included |
| P | Nominal cell size appropriate for model detail (generally 200 feet, minimum 50 feet for refinement areas) |
| P | Computation points are at default locations or changes documented |
| P | Mesh Cell and Face tolerances are default or changes documented |
| F P | No mesh errors |
| P | Cell face relationships are calculated |
| Breaklines |  |
| F P | Breaklines are consistent with the terrain data |
| F P | Breaklines sufficiently detailed to capture features (thalweg, banks, berms, etc.) |
| P | Cells do not overlap terrain features where appropriate (i.e., unwanted leaking) |
| P | Breakline cell sizing and parameters are appropriate |
| F P | 2D Area Connections placed at applicable benchmarks, gages, or dams |
| F | 2D Area Connections set to use normal 2D equation domain for benchmarks and gages. Appropriate modeling approach used for dams. |
| Manning’s N-Values |  |
| P | Manning’s n-values associated to land cover data provided |
| P | Land Cover classifications are sufficiently descriptive within the data table |
| P | Manning’s n-value classifications generally match current aerial imagery |
| F P | Manning’s n dataset is associated with the geometry files |
| General |  |
| P | All model datasets each have unique descriptive names |

## Review Comments

Modeler to Check Box of Acknowledgement during Response to Comments:

| X | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

- There are 24 cell errors in the mesh, fix all cell errors. Can open the Geometry Editor in RAS to see locations where these errors exist.

Watearth Response (5/24/2024): The issue must be a result of either transfer of the model or version difference, because these errors are not showing up in our model. Below is a snip from the geometry editor:

Backcheck response (7/09/2024): I am still seeing errors upon opening the model. Here is a screenshot of what I see in Geometry Editor

.

When I zoom into a cell error shown, it does show the cell having more than 8 sides so the error seems accurate. I did compare the cells seen in geometry editor to the ones in Ras Mapper and they look to be slightly different with the ones in Ras Mapper not showing the issue of more than 8 sides. This discrepancy should be looked into since Geometry editor can’t be showing any errors when submitted. I’m happy to jump on a call to discuss if needed.

Also there is an issue showing in Ras Mapper on the 2D Flow Areas where it says “Mesh not created”. This may be contributing or related to the issue, but no red asterisks should be present in Ras Mapper so it needs to get addressed as well

Watearth Response (7/17/2024): Cell errors found in the geometry editor and RAS mapper are still not appearing within our model. AtkinsRéalis will schedule a review call to address these findings.

Backcheck 2 Response (7/24/2024): No more cell or mesh errors can be seen on my end. Comment has passed.

- There are multiple missing segments of streamlines throughout the model when streamlines should be continuous. Below are a couple of examples where segments are missing, first one is located just left of TOBA_301_BC3 boundary condition line and the second is just upstream of outlet TOYA_003_BC2. These are not the only locations where this occurs so please review entire model area for similar issues.

Watearth Response (5/24/2024): Will review the entire model and add stream centerlines where the segments are missing.

Watearth Response (6/20/2024): We added the missing stream segments to the model with their refinement regions. Will review the entire model and add stream centerlines where the segments are missing as shown in the snip below. Also the entire model is reviewed for missing breaklines and refinement regions.

Backcheck response (7/09/2024): Comment has passed.

- Relating to above comment, some streamlines don’t have refinement regions around the breakline. See above screenshot in previous comment for example circled in red. Would recommend being consistent for all stream breaklines and checking through entire model to make sure refinement regions cover all streamlines.

Watearth Response (6/20/2024): We reviewed the entire model and added stream centerlines where the segments are missing, added refinement regions where missing.

Backcheck response (7/09/2024): Comment has passed.

- Please provide more information in write-up about why 0.3 sqmi breaklines were chosen to be used instead of 1 sqmi that is stated in the TWDB guidelines.

Watearth Response (5/24/2024): AtkinsRealis will provide an explanation for the use of 0.3 sqmi streamlines for the modeling.

Backcheck response (7/09/2024): Pending the review of updated AtkinsRealis report.

Backcheck 2 Response (7/24/2024): The comment can be passed for Milestone 1, with the final BLE report (for this HUC, as opposed to Salt Draw) to be reviewed/confirmed during Milestone 2.

M2 AtkinsRéalis response (10/03/2024): Adding breaklines along streamlines reduces nominal cell spacing and produces more continuous floodplains. Lower stream break line threshold will extend the mapping continuity into upstream reaches, although it increases number of cells and lead to longer model run time. So, the 0.3 sq mi threshold was chosen to balance computational efficiency with model refinement along larger/more defined stream corridors. We added discussion in BLE report subsection 2.8.2 regarding this item.

M2 Reviewer Response (10/10/2024): Report updated with sufficient discussion, comment is passed.

- Some terrain modifications are missing. Review entire model to check for terrain modifications where streamlines are crossing roadways and other necessary locations. Also be sure to include information about this approach at the breaklines in the write-up.

Watearth Response (6/20/2024): We reviewed the entire model and added hydro-connectors where missing.

Backcheck response (7/09/2024): There are still a few locations I’ve spotted that would benefit from terrain modifications and relieve significant ponding where culverts are located. I have created a shapefile called QC_TerrainMods.shp that marks these locations.

Watearth Response (7/17/2024): We reviewed the entire model and added terrain modifications based on additional findings and the supplied QC_TerrainMods.shp.

Backcheck 2 Response (7/24/2024): Comment has passed.

M2 AtkinsRéalis response (10/03/2024): In subsection 2.8.2 of the BLE report we have added discussion of terrain modifications with graphic.

M2 Reviewer Response (10/10/2024): Report updated with sufficient discussion, comment is passed.

- 2D connections for Out2_TOYA_003, USGS08431700, and USGS08431800 should be re-examined and adjusted to not cross any disconnected or offline 1% flow from other streams, as well as be more perpendicular to the floodplain. For Out2_TOYA_003, would also suggest moving downstream of confluence if it’s not too far away from gage location.

Watearth Response (6/20/2024): The SA2D connections are drawn based on 500-year run results so the connections cover the entire 500-year flood plain. We split the SA2D connection for Barilla Draw so the disconnected stream has a separate 2D connection. We also added another SA2D connection for an unnamed tributary after the addition of some more stream segments to the northwestern edge of the model.

Backcheck response (7/09/2024): For 2D connections being used to validate flows for calibration, they should be drawn perpendicular to flow and capture the 1pct floodplain for that location. If you’re drawing based off the 500-yr floodplain and it becomes a shared floodplain like OF2_BRLLDRW_003 then you’re capturing more flow on the 1pct that shouldn’t be captured for validation flows. I would suggest picking a location more upstream or downstream where capturing the 500-yr floodplain doesn’t involve capturing offline flow from another stream for the 1pct to ensure validation flows are accurate. Looking at the same 2D connection mentioned, the left overbank side of the connection becomes bent and is no longer perpendicular to flow like guidance states. Would suggest moving this connection more upstream where it can be drawn perpendicular while capturing the flow for all events and no offline flow is being considered. I’m happy to jump on a call to discuss if needed.

Watearth Response (7/17/2024): 2D connection was split, moved downstream to capture the 1pct floodplain, and drawn perpendicular to the flow.

Backcheck 2 Response (7/25/2024): Based off call on 7/25/24, the 2D connection was not moved upstream at all due to this connection being located at the boundary of the DS model and not necessary for validation. Also the approach of following the contour line to draw their 2D connection rather than a straight perpendicular line across the floodplain was discussed. These methods are reasonable but it’s recommended to include a write-up outlining this approach in the report. The comment not passed yet, as the 2D connection needs to be enforced at this location as well.  Since model needs to be re-run due to % Impervious issue (see Comment #10 in this section, below), recommend making this correction as well with final model run. If possible to add the additional write-up explaining the approach and provide that for the M1 ITR reviewer to confirm as well, please do.  Otherwise, the comment can be cleared once the 2D connection is enforced and ITR team will review the final BLE report, to include the above-mentioned write-up, during Milestone 2.

Watearth Response (7/28/2024): The Sa/2D connection is now enforced and screenshot from the updated model is shown below. In flat areas, where the floodplains are wide and divided and flow travels in multiple directions, the WSE contour helps guide placement of cross-sections. We will add discussion regarding this item in the BLE report, which will be submitted as part of the M2 submittal.

Backcheck 3 response (8/23/2024): Breakline is enforced, so comment has passed. As stated previously, we will review the final report and check for additional discussion in M2.

M2 AtkinsRéalis response (10/03/2024): In subsection 2.7.4 of the BLE report we have added detailed discussion of SA/2D connection placement.

M2 Reviewer Response (10/10/2024): Report updated with sufficient discussion, comment is passed.

- When trying to see Final N value layer in RAS Mapper, no values or layer shows up. This could be an issue on reviewer’s end but please investigate to make sure n values are being applied properly and are available to view in the Final N Value layer.

Watearth Response (5/24/2024): We can see the final N value layer in RAS Mapper on our side. Might be a version issue, or due to transfer of large models.

Backcheck response (7/09/2024): Still unable to view the Final Manning N layer on my end. This is what I’m seeing, where the layer is turned on and should be viewable but nothing is showing up. It should be viewable so it can be reviewed, I’m happy to jump on a call to discuss if needed.

Watearth Response (7/17/2024): AtkinsRéalis will schedule a review call to address these findings.

Backcheck 2 Response (7/24/2024): Final N value layer can now be viewed. Comment has passed.

- Would suggest following naming convention for model and plans that is outlined in the TWDB guidance document.

Watearth Response (6/20/2024): We revised the model and plan names following the latest TWDB guidelines.

Backcheck response (7/09/2024): Comment has passed.

- There are no refinement regions for urban areas of Fort Davis CDP and Balmorhea City which both fall in this model area. Please update or provide documentation on why these aren’t included.

Watearth Response (6/10/2024): We added refinement regions for additional urban areas.

Backcheck response (7/09/2024): Comment has passed.

- New Comment: % Impervious is double accounting for CN values in developed areas. ITR team understands that the Atkins team is aware of this and will decide on a method to correcting this and submit correction.

Watearth Response (7/27/2024): The CN values are corrected in the HEC-RAS model and now use the same CN for low, medium, and high developed intensity land use categories with varying impervious percentages defined in the TWDB guidance. Please see the screenshot below from the updated model showing  corrected AMCI CN values.  With this update we have addressed the double counting of impervious percentages. First screenshot shows AMCI CN values and % imperviousness values rom updated model.

- Backcheck3 response (8/23/2024): Comment has passed.

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to confirm with Pass/Fail)

| Boundary Review |  |
| --- | --- |
| F P | External boundaries connected to 2D mesh |
| P | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh |
| P | Boundary conditions have a hydrograph or elevation/slope data assigned |
| N/A | Input hydrographs consistent with hydrologic data (DSS, manual entry) |
| P | Precipitation data is consistent with hydrologic data (DSS, manual entry) |
| P | Downstream boundary consistent with ground slope or known downstream WSEL |
| N/A | Flow hydrographs applied at appropriate locations |
| N/A | Energy grade slope for hydrograph distribution is reasonable |
| N/A | Internal rating curves appropriate and documented for control structures (if applicable) |
|  | Modeling Tie-ins: Check for flow continuity between upstream/downstream model domains for all profiles (e.g. matching flow hydrographs) |

## Review Comments

Modeler to Check Box of Acknowledgement during Response to Comments:

| X | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

- Check that the boundary condition is connected to the mesh, could be a technical issue on reviewer’s part but usually there is a bold border around the mesh where it’s connected and not seeing that.

Watearth Response (5/24/2024): The boundary conditions are connected to the mesh. Might be a version issue or due to transfer of the model.

Backcheck response(7/09/2024): Similar to the responses above, I’m still not seeing the boundary conditions connected to the mesh. I can’t pass until I can review and confirm they are connected on my end. This is what I’m currently seeing in Ras Mapper where the drawn boundary condition line exists but no indication it’s connected to the perimeter

Watearth Response (7/17/2024): AtkinsRéalis will schedule a review call to address these findings.

Backcheck 2 Response (7/24/2024): The latest model submitted now has no issue with the boundary condition connectivity. Comment has passed.

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): | 10pct | 0.2% ACE (500-year): | 0.2pct |
| 4% ACE (25-year): | 4pct | 1% plus: | 1plu |
| 2% ACE (50-year): | 2pct | 1% minus: | 1min |
| 1% ACE (100-year): | 1pct |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: | Advanced time step based on Courant condition (7.5-60 Seconds) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: | Min: 0.45 | Max: 2 | Simulation Length: |  | hours |
| Computation Interval: | 15-60 | seconds | Model Runtime: | 12 | hours |
| Hydrograph Output Interval: | 10 | minutes | Mapping Output Interval: | 10 | minutes |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Execution Review |  |
| --- | --- |
| P | Appropriate 2D parameters are used (Boundary Conditions, Time Step, Simulation Length, etc.) |
| P | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks |
| P | Model runtime is reasonable (< 12 hours, but dependent upon model size) |

## Review Comments

Modeler to Check Box of Acknowledgement during Response to Comments:

| X | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

- Note:  Please be sure the specs in table above are accurate compared to the model settings when resubmitted.

Watearth Response (6/20/2024): The specs in the table above are the same ones as the model.

Backcheck response (7/09/2024): Comment has passed.

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: | N/A |
| Methodology | Regression flows were used in the absence of gage data and FIS flows; Draft InFIRM report was used to develop the inflow hydrograph for O.H. Ivie reservoir, pool frequency information obtained from RFA report is leveraged for reservoir modeling. Refer to the BLE report and the Model Overview and Results Spreadsheet. |
| Removal of Outliers and Justification | N/A |
| Review of Appropriateness of Gage Analysis results: - Check if there is a control structure upstream build in period of record, - Is the period of record sufficient, - Are there any gaps in the period of record, - Q vs A plot included - Wide confidence bands relative to flows? |  |
| Validation Notes (describe parameter adjustments): Refer to the BLE Report and the MidCo Model Overview and Results.xlsx |  |
| Other: More details included in the hydraulics technical memo. |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Computation Checks |  |
| --- | --- |
| P | Model runs with no cell errors |
| P | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |
| Results Spot Checks Check at all critical structures, major confluences, a handful of headwater tributaries, downstream end, at benchmarks, and a handful of randomly selected locations. |  |
| P | No visible spikes in WSEL visibly present with a single cell |
| F P | WSEL and depth grids appear reasonable (no artificial dips or spikes) |
| F P | Visual verification flooding is connected downstream for all significant contributing areas (> 64 acres) |
| P | Velocity grids appear reasonable (no unusually high velocity areas or Courant values greater than 2) |
| P | No crossing profiles between all events |
| P | Inflow and outflow hydrograph timing is reasonable as compared to the peak |
| P | Spills into/from adjacent HUCs appropriately accounted for |
| P | WSEL tie-in between model domains and HUCs along major rivers/streams where inflows are applied |
| Results Full Checks |  |
| P | Computational time step meets ideal Courant condition |
| P | Diffusion Wave: V∆T∆X ≤2 (Up to 5) (Results are unit-less) |
| P | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |
| F P | Hydrographs at internal and external boundaries are smooth |
| P | Contributing areas are contained with 2D mesh or accounted for with inflow hydrographs |
| P | No flow trapped at 2D boundary |
| F P | Visual verification mapping is reasonable compared to effective |
| Flow Verification |  |
| P | Peak flow gage analysis performed on all gages in the watershed with sufficient data |
| P | All gages used have an appropriate period of record (10 to 20+ years) |
| P | All gages used have no extensive gaps in record which would impact the analysis |
| P | Gage analysis results validated to other sources (Regression, USGS, etc.) |
| P | Results at applicable benchmarks justified (gages, HWMs, effective BFEs) |
| P | Peak discharges comparable to gages, effective/prior studies, regional regression equations |

## Review Comments

Modeler to Check Box of Acknowledgement during Response to Comments:

| X | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

- Multiple tributaries throughout entire model have unusually low flow or no flow at all. Terrain is very flat in this area of New Mexico but these floodplains don’t really seem reasonable. Please review entire model and investigate these instances of no flow along streams. Also, if appropriate, plan to provide some discussion/explanation in appropriate section of the write-up/report.

Watearth Response (6/20/2024): The flow is very low in the watershed due to the location as well as very flat terrains. The Davis Mountains at the upstream provide some steep slopes which is challenging for a large meshed model (due to large size of the watershed being modeled). This is causing some discontinuity in the flood plains in the model. Will provide a discussion about challenges faced modeling such an arid and flat watershed in the final report.

Backcheck response (7/09/2024): Response is acceptable. Comment passing is pending receipt of the final Salt Draw report, or revised Toyah memo write-up, that includes this discussion.

Backcheck 2 Response (7/25/2024): Confirming that this comment is passed for Milestone 1.  Final BLE report (for this HUC, as opposed to Salt Draw) will be reviewed and confirmed complete for Milestone 2.

M2 AtkinsRéalis response (10/03/2024): In subsection 3.1 of the BLE report we have added validation discussion and compared BLE flows versus gage analysis and discharge- drainage area curves, which are generally within validation range in Toyah HUC8. Generally low flows are attributed to arid conditions, low rainfall totals, high infiltration rates, flat topography in portions of the watershed etc. Additionally, flows along smaller tributaries were lowered to obtain valid flows at downstream gages on larger tributaries and Pecos River. Refer to subsection 3.1 of the BLE report for details. In section 3.2 we added discussion of excessive depths in mountainous areas, discontinuities along WTR_LN, validation challenges.

M2 Reviewer Response (10/10/2024): Report updated with sufficient discussion, comment is passed.

- Outflow hydrograph seen in DSS viewer for Out1_TOYA_003 becomes extremely jagged at end of simulation. Hydrograph needs to be smooth, would suggest adjusting simulation time to cut off earlier.

Watearth Response (6/10/2024): After the revisions and re-running of all plans for the model, there are no longer oscillations in the output hydrographs.

Backcheck response (7/09/2024): Comment has passed.

- Additional comment: Note that while the Depth/WSE rasters provided for the watershed area are viewable, the diff rasters provided are having issues when opening. When I try it just shows blank squares in ArcMap with no data viewable. There was also no WSE raster provided for the 1pct, and no Depth or WSE raster provided for the 4pct. At this time, tie-ins cannot be verified.

Watearth Response (7/17/2024): Depth/WSE rasters have been provided for all cases. Difference rasters show as blank squares when first loading into ArcGIS pro, but through legend edits can be viewed normally.

Backcheck 2 Response (7/25/2024): Tie-ins have been reviewed and look acceptable. Comment has passed.

# 7 – M1.5 Minor Post Processed Grids

## Toyah HUC-8 M1.5 Review Comments – 8/25/2024

The comments below must be addressed, and revised M1.5 minor post processed grids resubmitted in order to allow TWDB to count Independence as “SMA”:

- No mapping threshold has been identified. No ReadMe .txt files have been submitted.

M2 AtkinsRéalis response (10/03/2024): Inland mapping threshold metric was applied for this watershed. A readme file was submitted on 08/28/2024 following initial M1.5 mapping submittal package.

M2 Reviewer Response (10/10/2024): Passed.

The notes below are for documentation and to potentially alert the production team to issue that may need to be addressed during Milestone 2 final BLE map production – they do NOT require response or revised submittal for this M1.5 process:

- Reviewer notes/confirmation

-M1 backcheck is complete and comments are closed out – Noted

-All files (other than documentation of approach) are present and viewable – Noted

-Please confirm during M2 production that inland threshold is used in processing final mapping.  It was noted through use of an in house script that some areas smaller than approximately 0.33-acres exist in the minor post processed depth grids, however, there were few enough that its likely due to clipping. Please just be aware during final map production.

Atkins Response 09/30/2024:  The inland threshold was used, all polygons smaller than 0.33 acres are due to data clean up: clipping, smoothing, etc.

M2 Reviewer Response (10/10/2024): Passed.

-Extents of 1% WSE and Depth grids align, and extents of 0.2% WSE and Depth grids align – Noted

-Full mapping exists through entire HUC8 – Noted

-Depth Rasters – During review it was noted that the 10pct depth minor post processed grid has viewing errors. It exists, but there are zoom levels where its not viewable. This was fixed on our end by deleting and then recreating pyramids for this grid. – Noted

-WSEL Rasters – During review it was noted that the 1pct WSE minor post processed grid has viewing errors. It exists, but there are zoom levels where its not viewable. This was fixed on our end by deleting and then recreating pyramids for this grid. – Noted

# 7 – Mapping Results – MILESTONE 2

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
| P | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| P | All features fully populated per current FEMA guidelines |
| P | All features aligned and clipped to USGS HUC-8 boundary including: -Mapping should extend through entire HUC-8 -No mapping beyond HUC-8 boundary |
| P | Mapping (FLD_HAZ_AR) consistent between depth and WSE grids to polygons for all three required mapped events. The depth grid, water surface elevation grid and floodplain polygon extents must match per FEMA requirements, including the filling of holes, spillways, lake/pooled areas. |
| P | Containment within Events: (10% floodplain contained within 1% and 0.2%) |
|  | Centerline Containment within the 1% floodplain extents |
| P | Holes filled in polygon, depth and WSE grids: Holes Filled within the contiguous floodplain anything less than or equal to 2 acres in the 1- and 0.2% floodplain extents and anything less than or equal to 0.5 acres in the 10% floodplain extents. This should be clearly documented in the report. |
| P | Tie-ins Clarification: Tie-Ins realted to WSEL must be within 0.5ft for the following scenarios: 1) with currently studied HUCs (within the Team), 2) 2D HUCs available on the EBFE viewer, 3) Model Domain Tie-Ins must be 0.5 feet for all reoccurrence intervals 4)FEMA requires water surface elevation tie-ins within 0.5 ft for the mapped event of 10%, 1% and 0.2%. If mapping tie ins cannot be achieved: 1)Adjacent study not completed, 2) Adjacent study completed in 1D, or 3) Adjacent study completed with different modeling and mapping methodology causing tie in achievement to be difficult 1) Need to add AOMI area as AOMI_TYP = NP and AOMI_INFO = Spill area. In need of further engineering analysis. Where complex mapping tie ins between domains/HUCs exist, prior coordination with TWDB at M1 is recommended. (Ex. Braided channels) Please verify for Model Tie-Ins (Flow): o Each event needs to achieve flow continuity at the SA/2D connection. (E.g. Flow hydrographs must match at SA 2D connection between model domains). o A SA/2D connection line may be used to obtain a flow hydrograph between two model domains. The flow hydrograph obtained from the SA/2D connection line should match the Inflow Hydrograph Boundary condition to the downstream model domain. Please verify for Model/ Mapping Tie-Ins (WSEL): o Mapping tie ins (WSEL, Depth) are required for all modeled events across the entire floodplain’s width at one location in the model’s overlap region. This is validated using a raster calculation between both models’ WSEL grids. You must meet the WSEL tie in requirements and the SA/2D connection flow continuity between models. The WSEL tie ins requirement of ±0.5 ft. must be met in all model profiles (10, 25, 50, 100, and 500 yr.) and for 10, 100 and 500-yr for mapping products. Review of fringe mapping tie ins between study HUCs and other adjacent HUCs are within the ±0.5 ft. Both Model domains/HUC boundaries need to be in the ±0.5 ft. |
| P | Waterbodies Added (Lakes/Ponding Areas/Reservoirs) Water bodies of lake and reservoir inundation areas of 20 acres or greater or named (NHD waterbodies dataset) should be merged into final floodplains (FLD_HAZ_AR) and water areas (WTR_AR). |
| P | Boundary interfaces align matching or identified as AOMI |
| P | Centerline Containment (WTR_LN) within 1% floodplain extents |
| P | Schema |
| P | Review that DTL_STUD_AR and DTL_STUD_LN to ensure coverage of the NFHL effective detailed studies. |
| P | Reviewed that BFE feature values against the 1% annual chance event (ACE) water surface elevation (WSE) grid within +/- 1 foot |

## Review Comments

Modeler to Check Box of Acknowledgement during Response to Comments:

| X | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

- Please note that it doesn’t appear that the most up-to-date version of this QAQC ITR checklist has been used in the submittal, please plan to transfer all information from this copy to the updated/correct version upon resubmittal of Milestone 2 for bachcheck.

Response 20250423:  QC Document Updated

Backcheck – Approved, EDS

- CNMS – S_Studies_Ln is empty

Response 20250423: - S_Studies_Ln has been populated.

Backcheck – Approved, EDS

- All feature classes VERSION_ID field should be named as ‘BLE_MMYYYY’ based on the submittal date (will need to be updated at final approval/submittal)

Response 20250423: Agreed, no changes made per discussed AECOM on follow-up Toyah call

Backcheck – Approved, EDS

- DTL_STUD_LN – Effective date for panel 4805370001B should be 9/16/1988, SOURCE_CIT = FIRM1

Response 20250423: Adjusted to 9/16/1988.

Backcheck – Approved, EDS

- BFEs should be split at the DTL_STUD_AR boundary.  Those within the boundary will be ‘NOT MAPPED’, those outside of the boundary should be ‘MAPPED’.

Response 20250423: Split BFEs by DSA boundary.  Those inside are now ‘NOT MAPPED’.

Backcheck – Approved, EDS

- See ‘Centerline_Containment’ for line features that should be corrected to stay within the 1% SFHA.

Response 20250423: Adjusted geometry so centerline stays within Zone A.  A few sections beyond the 3 callouts were adjusted to, as they just barely existed Zone A.

Backcheck – Approved, EDS

- There are locations where there is a visible waterbody in the imagery that the 1% does not fully inundate, some of these are waterbodies found in the WTR_AR feature class, others are not.  Adjustments should be made to the 1% (see bookmarks, review any other locations and apply changes where necessary).

Response 20250423: FLD_HAZ_AR adjusted so that waterbodies are completely inundated.  WSE and Depth grids are also updated.

Backcheck – Approved, EDS

- Review WSE/Depth Containment check.  There is an isolated cell in the 1% grids where no 1% SFHA exists. If WSE grids are revised based on comment above/below, this will need to be checked again.

Response 20250423: Clipped out the single cell from Depth and WSE 1%.

Backcheck – Approved, EDS

- Review BFE_2D_Testpoints.  39% of the test points fail.  This is likely due to steep slope, but should be reviewed.

Response 20250403: The test points were reviewed and all failed areas are related to steep slopes and cupping. As part of a review, additional thresholds were reviewed and the results are as follows:

Total Points:  1,282,148

Total above abs|1|:  293,286, 22.9%

Total above abs|2|:  82,584, 6.4%

Total above abs|5|:  5,507, 0.43%

Total above abs|10|:  623, 0.05%

All issues seem to be related to cupping or very thin raster areas.

Backcheck – Approved, EDS

- The submittal checklist above (Page 2 of this document) for M2 submittal (section 7/section 8) is incomplete.

AR response 20250508: The submittal checklist for M@ submittal completed

- Modeler initials and submittal date need to be filled in for Mapping Review section at the beginning of this document

Response 20250423:  QC Document Updated

Backcheck – Approved, EDS

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
| P | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
| P | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
| F | Technical 2D BLE report documents all assumptions, special considerations, etc. |
| P | Flow validation and difference discussed in report |
| P | Flow comparisons discussed in report |
| Hydrologic Model |  |
| P | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
| P | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| P | All features fully populated per current FEMA guidelines |
| CNMS |  |
| P | Updated correctly with new centerlines |
| P | A1-A5 checks complete |
| P | CNMS Post- Processing Complete |
| HAZUS |  |
| P | Run per FEMA guidance |

## Review Comments

Hazus

- S_FRAC_Ar  should not be clipped by the watershed.  According to the Flood Risk Assessment Guidance, exported Hazus data should include any census blocks that intersect the watershed and should not be clipped.

Response 20250423: FRAC is now selected by census blocks that intersect the HUC8 boundary, but not clipped by the boundary.

Backcheck – Approved, EDS

- Additionally, the census blocks that were considered inventory of the project footprint / watershed, but were not considered a loss, should have their loss populated as -8888 rather than 0.

Response 20250423: the census blocks that were considered inventory of the project footprint / watershed, but were not considered a loss populated as -8888 rather than 0

Backcheck – Approved, EDS

File Structure and Naming

- Modeler initials and submittal date need to be filled in for Completeness Review section at the beginning of the document

Response 20250423:  QC Document Updated

Backcheck – Approved, EDS

Report

- There are a couple of comments added into the report to address.

Backcheck – Previous comments are approved [EDS]. There are a few new comments added to the report pointing out a couple changes that were made in other reports that didn’t get changed in the Toyah report [JW].

AR response 20250508: Report updated

Flood Risk Database

- See comments in Section 7

CNMS

- S_Studies_Ln is empty

Response 20250423:  Populated.

Backcheck – Approved, EDS

- Unclear if A1-A4 is complete, A5 in the report is incomplete

Response 20250423: Report is updated with this info.

Backcheck – Approved, EDS

- Post Processing is incomplete with the empty S_Studies_Ln

Response 20250423:  Populated.

Backcheck – Approved, EDS

- CNMS references/section incomplete in the report.

Response 20250423: Report is updated with this info.

Backcheck – Approved, EDS

Terrain

- Source_Index does not fully cover all tiles delivered in ‘Bare_Earth_DEM’ raster. Please add necessary tiles to cover.

AR response 20250508:  Source_Index updated to fully cover ‘Bare_Earth_DEM’ raster

- S_Submittal_Info is much larger than the extents of the Source_Index and raster deliverable in ‘Bare_Earth_DEM’.  Please consider clipping to the revised Source_Index (see comment above)

AR response 20250508:  Updated  S_Submittal_Info to match the extent of Source_Index

- S_Submittal_Info should also follow FEMA Data Capture Guidance where TERRAIN CAPTURE tasks should not have overlapping polygons from different sources.

AR response 20250508:  Overlapping areas removed

- Metadata comments

  - The <pubdate> in the <citation> and <timeperd> sections should be set to the MIP Submission Date.

AR response 20250508: Updated

  - Update bounding coordinates based on changes made in comments above about Source_Index and S_Submittal_Info

AR response 20250508: Bounding coordinates updated.

  - <placekey> entries are missing some values.  Please ensure the REGION, STATE, COUNTY, COUNTY-FIPS, COMMUNITY, FEMA-CID and HUC8 code is entered for each community in the purchase geography.

AR response 20250508:  Placekey entries updated.

  - Replace miphelp@riskmapcds.com in all locations with updated MIPHelp address of fema-riskmap-ithelp@fema.dhs.gov

AR response 20250508:  Updated

  - <srccontr> text should be input in the CONTRIB field in L_Source_Cit for each TOPO#

AR response 20250508:  Updated

  - In the <eainfo> section, an entry should be added for the Source_Index deliverable

AR response 20250508:  Updated

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – [add company name] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QC Manager |  | Date |  | Task Lead |  | Date |
| QC Reviewer |  | Date |  | Project Manager |  | Date |