# Texas 2D BLE Analysis QA/QC Checklist

# Project Information

| HUC-8 Watershed Name & ID: | Pecos Watershed, including portions of Landreth-Monument Draws (13070007), Coyanosa-Hackberry Draws (13070006), and Lower Pecos-Red Bluff Reservoir (13070001) |
| --- | --- |
| HUC-10 Watershed Name & ID: | Pecos Watershed, including portions of various HUC-10 Watersheds |
| Modeler: | EDGE Engineering |
| Reviewer: | AECOM |

## Terrain and Modeling Review (QC Items 1-6)

| Modeler Initials: | CDK | Reviewer Initials: | JW |
| --- | --- | --- | --- |
| Submittal Date: | May 17, 2024 | Review Submitted Date: | 6/6/24 |
| Review Response Date: | 7/30/24 | Backcheck Complete Date: |  |

## Mapping Review (QC Item 7) – not included in this submittal

| Modeler Initials: |  | Reviewer Initials: |  |
| --- | --- | --- | --- |
| Submittal Date: |  | Review Submitted Date: |  |
| Review Response Date: |  | Backcheck Complete Date: |  |

## Completeness Review (QC Item 8) – not included in this submittal

| Modeler Initials: |  | Reviewer Initials: |  |
| --- | --- | --- | --- |
| Submittal Date: |  | Review Submitted Date: |  |
| Review Response Date: |  | Backcheck Complete Date: |  |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X | 2D BLE |  | X | 10% ACE (10-year) | X | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | X | 4% ACE (25-year) | X | 1% plus |
|  |  |  | X | 2% ACE (50-year) | X | 1% minus |
|  |  |  | X | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

Reviewer to indicate if Technical Review items generally comply using a “Pass”. If the technical review observes an issue of noncompliance or an item that requires additional attention, an “Fail” should be utilized.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
| X | Location Map (Mapper): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map |
| X | GIS Data (Spatial and Mapper) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| Technical Report / Documentation (QC items 1-6) |  |
| X | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
| X | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| X | Internal QA/QC Documentation: comments and responses |
| X | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
| X | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
| X (Report) | Hydrologic Data and Parameter Tables (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
| X (Report) | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
| X | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files) |
| X (Report) | Hydraulic Data and Parameter Tables (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
| X | Manning’s n-value table |
| X | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
| X (Report) | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Mapping (QC item 7) |  |
|  | Flood risk database |
|  | Modeler to complete topology validation as outlines in the guidelines |
|  | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
|  | Technical BLE Report |
|  | Final cleaned hydrologic model |
|  | Final cleaned hydraulic model |
|  | Flood risk database |
|  | CMNS updates |
|  | HAZUS results |
|  | Additional supplemental data |

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): | TNRIS LiDAR |
| --- | --- |
| Data Gaps: | None |
| Source Resolution: | Cell size of 1 x 1 meter |
| Model Terrain Resolution: | Cell size of 5 x 5 feet |
| Terrain Horizontal Projection: | North American Datum (NAD) 1983 State Plane Texas Central (FIPS 4203) |
| Terrain Vertical Datum: | NAVD88 |
| Z Values: | Units of feet |
| Terrain Buffer: | 25,000 feet |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| General Review |  |
| --- | --- |
| P | LiDAR source matches the latest available data at time of project initiation |
| P | Large reservoir or lake extents included in model domain |
| P | Check visible seams, gaps or holes of LiDAR sources (if more than one source was used) |

## Review Comments

- There are two different terrains associated across the plans, some are associated with “Terrain” and some are with “Terrain_HydroEnforced”. Please set all plans to be associated with the same terrain or document in the report why there are different terrains being used.

Response: Agree. The geometry has been associated with “Terrain_HydroEnforced”, which is a clone of “Terrain” but with terrain modifications as described in the report.

Backcheck:

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-HMS v. 4.11 |
| --- | --- |
| Rainfall Source: | NOAA Atlas 14, Volume 11, Version 2.0 |
| Rainfall Duration: | 5-min, 10-min, 15-min, 30-min, 1-hr, 2-hr, 3-hr, 6-hr, 12-hr, 24-hr |
| Depth Selection: | Domain Average |
| Rainfall Temporal Distribution: | 24 hours, Frequency Storm |
| Areal Reduction: | None |
| Loss Methodology: | Spatially varied CN |
| Unit Hydrograph Transform Method: | N/A |
| Inflow Hydrograph Source: | Lower Pecos, Belding (Location 1), Belding (Location 2), Coyanosa (Location 1), Coyanosa (Location 2), Monahans |
| Other: |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: | Coyanosa Draw nr Ft Stockton (ID #08435800) |
| --- | --- |
| Stream Gage Name & ID: | Pecos Rv nr Girvin, TX (ID #08446500) |
| Stream Gage Name & ID: | Pecos Rv bl Grandfalls, TX (ID #08441500) (Period of record ends before 1987) |
| Stream Gage Name & ID: | Three Mile Mesa Ck nr Ft Stockston, TX (ID #08444400) |
| Stream Gage Name & ID: | Gages with either no data or less than 10-years of records – Courtney Ck Trib nr Ft. Stockton (ID #08436800) Pecos Rv at RR 1776 nr Grandfalls (ID #08437710) Pecos Rv nr Grandfalls, TX (ID #08438100) |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Excess Rainfall |  |
| --- | --- |
| P | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event or other appropriate rainfall application |
| P | CN values appropriately reflect land use and soils for entire 2D area boundary |
| P | Losses are correctly calculated to establish the excess rainfall hyetographs by checking land use, soils and infiltration layers or other relative information |
| P | Total precipitation depths for 1%+ and 1%- are calculated correctly |
| P | Excess rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) |
| P | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |
| P | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |
| Inflow Hydrographs |  |
| P | Shape and time-to-peak of any inflow hydrographs resemble past documented large storm events |
| P | Hydrographs reasonable when compared to the other return periods. The shapes should be similar, and hydrographs from different events should not intersect unless there is an engineering reason. Peak flows from a lower return period event should not be higher than a higher return period event |
| P | Contributing hydrographs from another RAS model applied at coincident locations (Provide locations in base map data) |
| General Hydrology |  |
| No | Any flow regulating dams in the model area? (Yes/No) |
| No | Dam modeling methodologies applied and explained? (Yes/No). If yes, provide discharge rating curve calculations and gage analysis documenting regulated versus unregulated records, if applicable. |
| N/A | Model aligns with documentation of dam modeling approaches. |
| Other Notes: |  |

## Review Comments

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.4.1 |
| --- | --- |
| Loss Methodology: | Spatially varied CN |
| Soils Source: | SSURGO |
| Land Cover Source: | NLCD 2019 |
| Manning's Roughness Source: | Table provided in Report |
| Nominal Mesh Cell Sizes: | 300’x300’ generally, 150’x150’ along channels and in refinement regions. |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| 2D Mesh |  |
| --- | --- |
| F | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included |
| P | Nominal cell size appropriate for model detail (generally 200 feet, minimum 50 feet for refinement areas) |
| P | Computation points are at default locations or changes documented |
| P | Mesh Cell and Face tolerances are default or changes documented |
| P | No mesh errors |
| P | Cell face relationships are calculated |
| Breaklines |  |
| F | Breaklines are consistent with the terrain data |
| F | Breaklines sufficiently detailed to capture features (thalweg, banks, berms, etc.) |
| P | Cells do not overlap terrain features where appropriate (i.e., unwanted leaking) |
| P | Breakline cell sizing and parameters are appropriate |
| P | 2D Area Connections placed at applicable benchmarks, gages, or dams |
| P | 2D Area Connections set to use normal 2D equation domain for benchmarks and gages. Appropriate modeling approach used for dams. |
| Manning’s N-Values |  |
| P | Manning’s n-values associated to land cover data provided |
| P | Land Cover classifications are sufficiently descriptive within the data table |
| F | Manning’s n-value classifications generally match current aerial imagery |
| P | Manning’s n dataset is associated with the geometry files |
| General |  |
| F | All model datasets each have unique descriptive names |

## Review Comments

- The extent of the model area does not seem to line up with the border of Landreth-Monument Draws HUC watershed. Please look into this and provide more documentation as to how the Pecos model area was delineated and why it slightly differs from the HUC8 watershed boundary. An example of the discrepancy I’m seeing is included below, where the Landreth-Monument Draw watershed boundary is in purple.

Response: Agree. The Imperial (Pecos) watershed boundary has been revised to ensure no gaps exist between the adjacent watersheds.

Backcheck:

- There are multiple locations where terrain modifications could be added along roads and embankments to allow flow to continue downstream. Would suggest to review model for similar instances to add terrain modifications to, especially focusing on where streamlines cross roads and embankments. Also be sure to include information about this approach at the breaklines in the write-up.

Response: Agree. The geometry was originally linked to the wrong terrain "Terrain", which did not contain any terrain modifications. The geometry has been changed to "Terrain.HydroEnforced", which contains terrain modifications. The entire study area has been looked over to ensure that appropriate crossings have been captured as terrain modifications.

Backcheck:

- There are instances throughout model where the are areas that don’t have cells aligned properly to the breaklines, see screenshot below for some examples. Would suggest reviewing model for similar instances to help improve the function of the breaklines.

Response: Agree. Breaklines have been revised throughout the model for better cell alignment.

Backcheck:

- Manning N values is set to 0.04 for all channel N values. Would suggest providing more details in the write-up about the method used and why there is only one channel N value used throughout entire model.

Response: Disagree. The Normal N-Value for “Clean, winding, some pools, and shoals” was used from Table 2.3 of the TWDB guidance. This value was selected as best representative for the Imperial study area. Using it in all model channels is appropriate for a BLE study and does not need to be discussed in the write-up.

Backcheck:

- Names for models and plans do not follow TWDB guidelines. Would suggest changing based off recommended naming conventions from these guidelines or from the updated Halff guidance. Please make sure to be consistent on type of naming with the Monument model and other models in Landreth-Monument Draw HUC.

Response: Agree. HEC-HMS and HEC-RAS models have been renamed to follow TWDB naming convention with the following exceptions agreed to by Atkins and TWDB: The first 4 letters of the HUC 8 name were used for naming and HUC-8 number last 3 digits for numbering.

Backcheck:

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to confirm with Pass/Fail)

| Boundary Review |  |
| --- | --- |
| P | External boundaries connected to 2D mesh |
| P | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh |
| F | Boundary conditions have a hydrograph or elevation/slope data assigned |
| P | Input hydrographs consistent with hydrologic data (DSS, manual entry) |
| P | Precipitation data is consistent with hydrologic data (DSS, manual entry) |
| P | Downstream boundary consistent with ground slope or known downstream WSEL |
| P | Flow hydrographs applied at appropriate locations |
| P | Energy grade slope for hydrograph distribution is reasonable |
| N/A | Internal rating curves appropriate and documented for control structures (if applicable) |

## Review Comments

- Please document about the use of internal boundary condition lines for all inflow hydrographs in the report.

Response: Agree. Section 1.2 has been revised to include this information.

Backcheck:

- Normal depth slopes for General BC lines in the 10year plan are 0.01 when every other event uses 0.001. Would suggest adjusting slopes on 10year event to be consistent with other plans, or provide documentation on why this plan uses different slope values.

Response: Agree.

Backcheck:

- DSS file does not have data for 50year, 100yearPlus, and 100yearMinus for any BCLINE. Please make sure all events are included in the DSS file.

Response: Agree. All events are now included in the DSS file.

Backcheck:

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): | IMPE_007_10pct | 0.2% ACE (500-year): | IMPE_007_0_2pct |
| 4% ACE (25-year): | IMPE_007_4pct | 1% plus: | IMPE_007_1plu |
| 2% ACE (50-year): | IMPE_007_2pct | 1% minus: | IMPE_007_1min |
| 1% ACE (100-year): | IMPE_007_1pct |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: | Advanced time step based on Courant condition (15-240 Seconds) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: | Min: 0.45 | Max: 2 | Simulation Length: | 336 | hours |
| Computation Interval: | 60 | seconds | Model Runtime: | 1-8 | hours |
| Hydrograph Output Interval: | 10 | minutes | Mapping Output Interval: | 10 | minutes |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Execution Review |  |
| --- | --- |
| P | Appropriate 2D parameters are used (Boundary Conditions, Time Step, Simulation Length, etc.) |
| P | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks |
| P | Model runtime is reasonable (< 12 hours, but dependent upon model size) |

## Review Comments

- Model run times are between 20-35 hours. This is an extremely massive model size so large runtime is expected. If possible, I would suggest looking into upstream models runtime to see if any simulation length can be trimmed at all. Right now it’s the reason Pecos has to run for 15 days and trimming this model’s simulation length would help improve overall runtime.

Response: Noted. The model runtime cannot be shortened because the upstream models (Monument, Monahans) have to run for 12-14 days to fully capture their peaks, and the same simulation time has to be used for Imperial to capture tie-ins with those watersheds.

Backcheck:

- Note: Values in Statistical Data above is not accurate, please update to match to reflect what is in the model.

Response: Agree.

Backcheck:

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: | HEC-SSP |
| Methodology | Schedule 13B |
| Removal of Outliers and Justification | N/A |
| Validation Notes (describe parameter adjustments): None |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Computation Checks |  |
| --- | --- |
| P | Model runs with no cell errors |
| P | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |
| Results Spot Checks Check at all critical structures, major confluences, a handful of headwater tributaries, downstream end, at benchmarks, and a handful of randomly selected locations. |  |
| P | No visible spikes in WSEL visibly present with a single cell |
| P | WSEL and depth grids appear reasonable (no artificial dips or spikes) |
| F | Visual verification flooding is connected downstream for all significant contributing areas (> 64 acres) |
| P | Velocity grids appear reasonable (no unusually high velocity areas or Courant values greater than 2) |
| P | No crossing profiles between all events |
| P | Inflow and outflow hydrograph timing is reasonable as compared to the peak |
| F | Spills into/from adjacent HUCs appropriately accounted for |
| Pending | WSEL tie-in between model domains and HUCs along major rivers/streams where inflows are applied |
| Results Full Checks |  |
| P | Computational time step meets ideal Courant condition |
| P | Diffusion Wave: V∆T∆X ≤2 (Up to 5) (Results are unit-less) |
| P | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |
| P | Hydrographs at internal and external boundaries are smooth |
| F | Contributing areas are contained with 2D mesh or accounted for with inflow hydrographs |
| P | No flow trapped at 2D boundary |
| F | Visual verification mapping is reasonable compared to effective |
| Flow Verification |  |
| Pending | Peak flow gage analysis performed on all gages in the watershed with sufficient data |
| Pending | All gages used have an appropriate period of record (10 to 20+ years) |
| Pending | All gages used have no extensive gaps in record which would impact the analysis |
| Pending | Gage analysis results validated to other sources (Regression, USGS, etc.) |
| F | Results at applicable benchmarks justified (gages, HWMs, effective BFEs) |
| F | Peak discharges comparable to gages, effective/prior studies, regional regression equations |

## Review Comments

- Multiple tributaries throughout entire model have unusually low flow or no flow at all. This was previously discussed over a call where it makes sense that the flat terrain, minimal rainfall and the use of ASC I losses are all playing a role in this. Please plan to provide some discussion/explanation in appropriate section of the write-up/report for these factors affecting the model and causing disconnected floodplains. Also be sure to review for any technical errors that might be causing lower flows.

Response: Agree. Flows in the Imperial watershed are generally very low, but seem reasonable for the area both qualitatively and quantitatively. See Section 1.3.6 in the memo which has been updated with a more detailed discussion of validation.

Backcheck:

- There are two inflows from Monahans that are not accounted for in the Pecos model, one near the Monahans mainstem inflow, and one near the left edge of the “General BC 8” boundary condition line. These are mentioned in the previous QC review for Monahans, so please see the Monahans QC document (Section 6, Review Comment #4 and Section 4, Review Comment #1) for more specifics on where these locations are.

Response: Partially Agree. An additional inflow “Inflow2_MONA_007_BC” from Monahans into Imperial (Pecos) has been added to the model near the mainstem inflow. However, no additional inflow from Monahans into Imperial (Pecos) near “General BC 8” was observed in the Monahans model (see screenshot below). There is no connected mapping at the apparent drainage outlet at that location for the 500-year event, and the flow is nearly 0 cfs. It is unlikely that this area is contributing flow to the Imperial (Pecos) model downstream.

Backcheck:

- The model area having discrepancies with the HUC8 border was commented on earlier, these discrepancies may not capture all contributing drainage area with 2D mesh.

Response: Agree. See response Section 3 “Hydraulic Geometry” Comment 1.

Backcheck:

- Noting that the model is using internal boundary conditions for inflow hydrograph locations from upstream models. The WSEL tie-ins and mapping with all upstream and downstream models will need to be reviewed in MS2.

Response: Noted.

Backcheck:

- Please provide the gage analysis documentation so it can be reviewed.

Response: Documentation of the gage analysis methods is included in the Salt-Draw report. Section 1.3.6 of the Imperial report has been updated to include the results of the gage analysis.

Backcheck:

- Based off the Pecos validation results table in the report, the model does not seem to be calibrated. Please confirm if calibration is still pending, and be sure to update the report with validation documentation.

Response: Agree. Validation section has been updated with revised results.

Backcheck:

# 7 – Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
|  | All features fully populated per current FEMA guidelines |
|  | Mapping consistent between grids to polygons for all three required mapped events |
|  | 10yr smaller than 100yr smaller than 500yr |
|  | Holes filled in polygon, depth and WSE grids |
|  | Boundary interfaces align matching or identified as AOMI |

## Review Comments

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
|  | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
|  | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
|  | Technical 2D BLE report documents all assumptions, special considerations, etc. |
|  | Flow validation and difference discussed in report |
|  | Flow comparisons discussed in report |
| Hydrologic Model |  |
|  | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
|  | All features fully populated per current FEMA guidelines |
| CNMS |  |
|  | Updated correctly with new centerlines |
|  | A1-A5 checks complete |
|  | CNMS Post- Processing Complete |
| HAZUS |  |
|  | Run per FEMA guidance |

## Review Comments

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – [add company name] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QC Manager |  | Date |  | Task Lead |  | Date |
| QC Reviewer |  | Date |  | Project Manager |  | Date |