# Texas 2D BLE Analysis QA/QC Checklist

# Project Information

| HUC-8 Watershed Name & ID: | Landreth-Monument Draws, 13070007 |
| --- | --- |
| HUC-10 Watershed Name & ID: | 1307000701, 1307000702, 1307000703, 1307000704, 1307000705, 1307000706 |
| Modeler: | Clint Kimball (EDGE Engineering) |
| Reviewer: |  |

## Terrain and Modeling Review (QC Items 1-6)

| Modeler Initials: | ck | Reviewer Initials: | JW |
| --- | --- | --- | --- |
| Submittal Date: | 5/1/2024 | Review Submitted Date: | 5/20/2024 |
| Review Response Date: | 7/17/2024 | Backcheck Complete Date: |  |

## Mapping Review (QC Item 7) – not included in this submittal

| Modeler Initials: |  | Reviewer Initials: |  |
| --- | --- | --- | --- |
| Submittal Date: |  | Review Submitted Date: |  |
| Review Response Date: |  | Backcheck Complete Date: |  |

## Completeness Review (QC Item 8) – not included in this submittal

| Modeler Initials: |  | Reviewer Initials: |  |
| --- | --- | --- | --- |
| Submittal Date: |  | Review Submitted Date: |  |
| Review Response Date: |  | Backcheck Complete Date: |  |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X | 2D BLE |  | X | 10% ACE (10-year) | X | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | X | 4% ACE (25-year) | X | 1% plus |
|  |  |  | X | 2% ACE (50-year) | X | 1% minus |
|  |  |  | X | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

Reviewer to indicate if Technical Review items generally comply using a “Pass”. If the technical review observes an issue of noncompliance or an item that requires additional attention, an “Fail” should be utilized.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
|  | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map |
| X | GIS Map (RAS Mapper) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| Technical Report / Documentation (QC items 1-6) |  |
| X | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
| X | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| X | Internal QA/QC Documentation: comments and responses |
|  | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
| X | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
|  | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
|  | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
| X | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files) |
|  | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
|  | Manning’s n-value table |
| X | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
|  | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Mapping (QC item 7) |  |
|  | Flood risk database |
|  | Modeler to complete topology validation as outlines in the guidelines |
|  | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
|  | Technical BLE Report |
|  | Final cleaned hydrologic model |
|  | Final cleaned hydraulic model |
|  | Flood risk database |
|  | CMNS updates |
|  | HAZUS results |
|  | Additional supplemental data |

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): | Please see technical memo for more detail: West Texas LiDAR (2019) and Pecos Dallas LiDAR (2019) |
| --- | --- |
| Data Gaps: | None |
| Source Resolution: | Cell size of 1 x 1 meter |
| Model Terrain Resolution: | Cell size of 10 x 10 feet |
| Terrain Horizontal Projection: | North American Datum (NAD) 1983, State Plane Texas Central, FIPS 4203 |
| Terrain Vertical Projection: | NAVD88 |
| Z Values: | Units of feet |
| Terrain Buffer: | 20,000 feet |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| General Review |  |
| --- | --- |
| P | LiDAR source matches the latest available data at time of project initiation |
| P | Large reservoir or lake extents included in model domain |
| P | Check visible seams, gaps or holes of LiDAR sources (if more than one source was used) |

## Review Comments

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-HMS 4.11 |
| --- | --- |
| Rainfall Source: | NOAA Atlas 14 |
| Rainfall Duration: | 24 hours |
| Depth Selection: | Domain Average |
| Rainfall Temporal Distribution: | Frequency Storm |
| Areal Reduction: | None |
| Loss Methodology: | N/A (losses applied to hydraulic model) |
| Unit Hydrograph Transform Method: | N/A |
| Inflow Hydrograph Source: |  |
| Other: |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: | Antelope Draw near Jal, NM (08436000) |
| --- | --- |
| Stream Gage Name & ID: | Monument Draw Trib near Monument, NM (08437620) |
| Stream Gage Name & ID: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Excess Rainfall |  |
| --- | --- |
| P | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event or other appropriate rainfall application |
| P | CN values appropriately reflect land use and soils for entire 2D area boundary |
| P | Losses are correctly calculated to establish the excess rainfall hyetographs by checking land use, soils and infiltration layers or other relative information |
| F | Total precipitation depths for 1%+ and 1%- are calculated correctly |
| P | Excess rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) |
| P | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |
| P | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |
| Inflow Hydrographs |  |
| N/A | Shape and time-to-peak of any inflow hydrographs resemble past documented large storm events |
| N/A | Hydrographs reasonable when compared to the other return periods. The shapes should be similar, and hydrographs from different events should not intersect unless there is an engineering reason. Peak flows from a lower return period event should not be higher than a higher return period event |
| N/A | Contributing hydrographs from another RAS model applied at coincident locations (Provide locations in base map data) |
| General Hydrology |  |
| No | Any flow regulating dams in the model area? (Yes/No) |
| No | Dam modeling methodologies applied and explained? (Yes/No). If yes, provide discharge rating curve calculations and gage analysis documenting regulated versus unregulated records, if applicable. |
| N/A | Model aligns with documentation of dam modeling approaches. |
| Other Notes: NA |  |

## Review Comments

- Unable to verify how 1%+ and 1%- precip depths were calculated. Please provide more detail in write-up about how these were calculated, report currently only mentions 10%, 4%, 2%, 1%, and 0.2%.

Response: Agree. More detail has been provided in the report.

Backcheck:

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.4.1 |
| --- | --- |
| Loss Methodology: | Spatially varied CN |
| Soils Source: | SSURGO |
| Land Cover Source: | NLCD 2019 |
| Manning's Roughness Source: | Assigned based on land use. |
| Nominal Mesh Cell Sizes: | 300 x300 except for channel refinement regions, breaklines and urban regions, where 150 ft x150 ft was considered. |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| 2D Mesh |  |
| --- | --- |
| F | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included |
| P | Nominal cell size appropriate for model detail (generally 200 feet, minimum 50 feet for refinement areas) |
| P | Computation points are at default locations or changes documented |
| P | Mesh Cell and Face tolerances are default or changes documented |
| P | No mesh errors |
| P | Cell face relationships are calculated |
| Breaklines |  |
| F | Breaklines are consistent with the terrain data |
| F | Breaklines sufficiently detailed to capture features (thalweg, banks, berms, etc.) |
| P | Cells do not overlap terrain features where appropriate (i.e., unwanted leaking) |
| P | Breakline cell sizing and parameters are appropriate |
| P | 2D Area Connections placed at applicable benchmarks, gages, or dams |
| P | 2D Area Connections set to use normal 2D equation domain for benchmarks and gages. Appropriate modeling approach used for dams. |
| Manning’s N-Values |  |
| P | Manning’s n-values associated to land cover data provided |
| P | Land Cover classifications are sufficiently descriptive within the data table |
| F | Manning’s n-value classifications generally match current aerial imagery |
| P | Manning’s n dataset is associated with the geometry files |
| General |  |
| F | All model datasets each have unique descriptive names |

## Review Comments

- The extent of the model area does not seem to line up with the border of Landreth-Monument Draws HUC8 watershed. Please look into this and provide more documentation as to how the Monument model area was delineated and why it differs from the HUC8 watershed boundary. An example of the discrepancy I’m seeing is included below, where the Landreth-Monument Draw watershed boundary is in blue. Please note that the model area also does not match the model domain file submitted by the AtkinsRealis team.

Response: Agree. The Monument watershed boundary has been revised to ensure no gaps exist between the Monument, Upper Pecos, and Monahans watersheds.

Backcheck:

- There are instances where the streamline isn’t aligned with the channel centerline indicated by terrain. One example is shown below at the outlet of the model. Review entire model for these instances and fix necessary locations. Please also note breakline guidance related to stream centerlines in the TWDB 2D BLE Guidelines and document any exceptions or deviations from the guidelines, if they will remain.

Response: Disagree. The stream centerlines used in the model were developed by AtkinsRealis and follow project requirements. Because the Monument study area generally lacks dendritic stream networks and channels, stream alignment to channels is not possible or reasonable. Flooding is generally disconnected and accumulates in an unusually high number of surface depressions without ever finding its way downstream, making channel flow largely irrelevant.

Backcheck:

- Some terrain modifications are missing. Review entire model to check for terrain modifications where streamlines are crossing roadways and other necessary locations. Also be sure to include information about this approach at the breaklines in the write-up.

Response: Disagree. Potential locations for terrain modifications were reviewed throughout the model using LiDAR, historic aerial imagery from multiple dates, and street view imagery to identify areas where a culvert exists that conveys water under the roadway, railroad, dam, levee, or other similar feature. Terrain modifications were only added where it was reasonable to assume that a culvert exists. Detailed information about this approach has been included in the breaklines section 1.3.2 in the report. If there is evidence that a culvert exists where a terrain modification has not been added, such as suggestions of scour, a headwall, or culvert opening in any imagery, please provide that evidence in the backcheck response.

Backcheck:

- Manning N values is set to 0.04 for all channel N values. Would suggest using the table provided in the TWDB guidance for channel N values, or provide more details in the write-up why there is only one channel N value used throughout entire model.

Response: Disagree. The Normal N-Value for “Clean, winding, some pools, and shoals” was used from Table 2.3 of the TWDB guidance. This value was selected as best representative for the Monument study area. Using it in all model channels is appropriate for a BLE study and does not need to be discussed in the write-up.

Backcheck:

- Names for models and plans do not follow TWDB guidelines. Would suggest changing based off recommended naming conventions.

Response: Agree. HEC-HMS and HEC-RAS models have been renamed to follow TWDB naming convention with the following exceptions agreed to by Atkins and TWDB: The first 4 letters of the HUC 8 name were used for naming and HUC-8 number last 3 digits for numbering.

Backcheck:

- Please provide more information in write-up about why 0.3 sqmi breaklines were chosen to be used instead of 1 sqmi that is stated in the TWDB guidelines.

Response: Disagree. The TWDB guidelines section 1.2.2 states that breaklines should be between 0.1 sq-mi and 1.0 sq-mi as appropriate. 0.3 sq-mi falls within that range. Please refer to the Salt Draw report for more information.

Backcheck:

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to confirm with Pass/Fail)

| Boundary Review |  |
| --- | --- |
| P | External boundaries connected to 2D mesh |
| P | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh |
| P | Boundary conditions have a hydrograph or elevation/slope data assigned |
| N/A | Input hydrographs consistent with hydrologic data (DSS, manual entry) |
| P | Precipitation data is consistent with hydrologic data (DSS, manual entry) |
| P | Downstream boundary consistent with ground slope or known downstream WSEL |
| N/A | Flow hydrographs applied at appropriate locations |
| N/A | Energy grade slope for hydrograph distribution is reasonable |
| N/A | Internal rating curves appropriate and documented for control structures (if applicable) |

## Review Comments

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): | 10pct | 0.2% ACE (500-year): | 0_2pct |
| 4% ACE (25-year): | 4pct | 1% plus: | 1plu |
| 2% ACE (50-year): | 2pct | 1% minus: | 1min |
| 1% ACE (100-year): | 1pct |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: | Advanced time step based on Courant condition (15-240 Seconds) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: | Min: 0.45 | Max: 2 | Simulation Length: | 336 | hours |
| Computation Interval: | 60 | seconds | Model Runtime: | 5-8 | hours |
| Hydrograph Output Interval: | 10 | minutes | Mapping Output Interval: | 10 | minutes |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Execution Review |  |
| --- | --- |
| P | Appropriate 2D parameters are used (Boundary Conditions, Time Step, Simulation Length, etc.) |
| P | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks |
| P | Model runtime is reasonable (< 12 hours, but dependent upon model size) |

## Review Comments

- Just noting that Statistical Data provided above is outdated for Hydrograph and Mapping Output Intervals. Above shows 15 minutes when they’re both set to 10 minutes in RAS model. Same for simulation length, 8 days instead of 48 hours. Please be sure this are accurate compared to the model settings when resubmitted.

Response: Agree. Table has been updated to reflect current model settings.

Backcheck:

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: | N/A |
| Methodology | Refer to the BLE report. |
| Removal of Outliers and Justification | N/A |
| Validation Notes (describe parameter adjustments): Refer to the BLE Report |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Computation Checks |  |
| --- | --- |
| P | Model runs with no cell errors |
| P | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |
| Results Spot Checks Check at all critical structures, major confluences, a handful of headwater tributaries, downstream end, at benchmarks, and a handful of randomly selected locations. |  |
| P | No visible spikes in WSEL visibly present with a single cell |
| F | WSEL and depth grids appear reasonable (no artificial dips or spikes) |
| F | Visual verification flooding is connected downstream for all significant contributing areas (> 64 acres) |
| P | Velocity grids appear reasonable (no unusually high velocity areas or Courant values greater than 2) |
| P | No crossing profiles between all events |
| P | Inflow and outflow hydrograph timing is reasonable as compared to the peak |
| P | Spills into/from adjacent HUCs appropriately accounted for |
| Pending | WSEL tie-in between model domains and HUCs along major rivers/streams where inflows are applied |
| Results Full Checks |  |
| P | Computational time step meets ideal Courant condition |
| P | Diffusion Wave: V∆T∆X ≤2 (Up to 5) (Results are unit-less) |
| P | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |
| P | Hydrographs at internal and external boundaries are smooth |
| F | Contributing areas are contained with 2D mesh or accounted for with inflow hydrographs |
| P | No flow trapped at 2D boundary |
| F | Visual verification mapping is reasonable compared to effective |
| Flow Verification |  |
| N/A | Peak flow gage analysis performed on all gages in the watershed with sufficient data |
| N/A | All gages used have an appropriate period of record (10 to 20+ years) |
| N/A | All gages used have no extensive gaps in record which would impact the analysis |
| N/A | Gage analysis results validated to other sources (Regression, USGS, etc.) |
| F | Results at applicable benchmarks justified (gages, HWMs, effective BFEs) |
| F | Peak discharges comparable to gages, effective/prior studies, regional regression equations |

## Review Comments

- Multiple tributaries throughout entire model have unusually low flow or no flow at all. Terrain is very flat in this area of New Mexico but these floodplains don’t really seem reasonable. Also the main channel is seeing minimal flow, especially for smaller plans like 10yr and 25yr. Overall, entire model seems to be having extremely low flows that do not seem reasonable for a HUC this big. Please review – if appropriate plan to provide some discussion/explanation in appropriate section of the write-up/report.

Response: Disagree. Flows in the Monument watershed are generally very low, but seem reasonable for the area both qualitatively and quantitatively. See Section 1.3.6 in the memo which has been updated with a more detailed discussion of validation.

Backcheck:

- The flows mentioned in above comment do not seem to be validated either. The report seems to have multiple locations where flow was compared against gages and some of those flows were in 10,000cfs range, is that graphic correct and applicable to Monument?  Please provide more information in write-up about where those locations in the graphic are located and the validation process in general.

Response: Agree. Model validation section has been updated to include comparison with BLE flows and historical gage data within the main channel.

Backcheck:

- The model area having discrepancies with the HUC8 border was commented on earlier, these discrepancies may not capture all contributing drainage area with 2D mesh.

Response: Noted. See response to related comment 1 in Section 3 “Hydraulic Geometry”.

Backcheck:

- Noting that the WSEL tie-in with downstream model will be checked in the backcheck of this review, status of pass/fail is pending.

Response: Noted.

Backcheck:

# 7 – Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
|  | All features fully populated per current FEMA guidelines |
|  | Mapping consistent between grids to polygons for all three required mapped events |
|  | 10yr smaller than 100yr smaller than 500yr |
|  | Holes filled in polygon, depth and WSE grids |
|  | Boundary interfaces align matching or identified as AOMI |

## Review Comments

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
|  | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
|  | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
|  | Technical 2D BLE report documents all assumptions, special considerations, etc. |
|  | Flow validation and difference discussed in report |
|  | Flow comparisons discussed in report |
| Hydrologic Model |  |
|  | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
|  | All features fully populated per current FEMA guidelines |
| CNMS |  |
|  | Updated correctly with new centerlines |
|  | A1-A5 checks complete |
|  | CNMS Post- Processing Complete |
| HAZUS |  |
|  | Run per FEMA guidance |

## Review Comments

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – [add company name] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QC Manager |  | Date |  | Task Lead |  | Date |
| QC Reviewer |  | Date |  | Project Manager |  | Date |